Condition Inspection Checklist

| Aircraft: N830EH Make: Evan Krueger Model: Sonex B Serial Number: 0849B | Airframe TT: X.X Engine TT: X.X Engine Tach: X.X | Start Date: XX/XX/XXXX End Date: XX/XX/XXXX Performed By: Evan Krueger |
| --- | --- | --- |

Administrative

| Inspection Item | Fail | Pass | Comments |
| --- | --- | --- | --- |
| Verify required documents (ARRROW) present in aircraft and not expired [AOH Section 8] |  |  |  |
| Verify aircraft identification plate, placards, and markings are present and legible [AOH Section 2] |  |  |  |
| Verify service bulletins, airworthiness directives, and service letters are up to date from manufacturers [N/A] |  |  |  |
| Update maintenance manuals to latest revisions [N/A] |  |  |  |
| Verify Dynon and Garmin firmwares are up-to-date [Dynon 102949-016 and Garmin 190-02207-A2] |  |  |  |

Powerplant

| Inspection Item | Fail | Pass | Comments |
| --- | --- | --- | --- |
| Inspect engine compartment (pre-cleaning) [JEM0002-12 Section 8.8] |  |  |  |
| Clean engine and engine compartment [JEM0002-12 Section 7.3] |  |  |  |
| Perform pre-inspection test run [JEM0002-12 Section 9.1] |  |  |  |
| Inspect engine compartment including torque seal on through bolts [JEM0002-12 Section 8.8] |  |  |  |
| Inspect cylinders for oil leakage, cracks, discoloration, or damage [JEM0002-12 Section 8.8] |  |  |  |
| Inspect ram air ducts and baffles [JEM0002-12 Section 8.8] |  |  |  |
| Replace spark plugs and adjust gaps [JEM0002-12 Section 3.4] |  |  |  |
| Check cylinder head bolt tension [JEM0002-12 Section 9.15] |  |  |  |
| Inspect top spring washer, valve springs, and rockers [JEM0002-12 Section 9.16] |  |  |  |
| Inspect hydraulic lifters and rockers [JEM0002-12 Section 9.20] |  |  |  |
| Perform compression leak down check [JEM0002-12 Section 9.18] |  |  | #1: /80 ¦ #2: /80 ¦ #3: /80 ¦ #4: /80 ¦ #5: /80 ¦ #6: /80 |
| Change oil and oil filter [JEM0002-12 Section 9.5] |  |  |  |
| Measure fluid in oil separator and empty [ACV-S03-01 Manual] |  |  | Fluid Drained: |
| Inspect and replace fuel filter [AC43.13 Section 8-36] |  |  |  |
| Inspect engine controls and linkages for damage and smooth function [JEM0002-12 Section 8.8] |  |  |  |
| Inspect AeroInjector for damage and rubber connector for cracking [ACV-C07 Manual] |  |  |  |
| Perform fuel flow rate test and fuel shut off valve test [JEM0002-12 Section 9.19] |  |  | Fuel flow: |
| Inspect fuel lines, fittings, and fire sleeves for condition and leaks [AC43.13 Section 8-31] |  |  |  |
| Inspect oil hoses, lines, and fittings for condition and leaks [AC43.13 Section 8-5] |  |  |  |
| Inspect air filter housing and replace air filter [ACV-R10-01 Manual] |  |  |  |
| Inspect firewall for damage, cracks, proper sealant, and cleanliness [JEM0002-12 Section 8.8] |  |  |  |
| Inspect electrical wiring [JEM0002-12 Section 8.8] |  |  |  |
| Inspect starter and master solenoid connections [JEM0002-12 Section 8.8] |  |  |  |
| Inspect engine mounts and hardware for degradation and cracking [JEM0002-12 Section 8.8] |  |  |  |
| Inspect intake and exhaust system for damage and wear. [JEM0002-12 Section 8.8] |  |  |  |
| Inspect exhaust system springs and tabs [AC43.13 Section 8-49] |  |  |  |
| Inspect flywheel screws and check torque [JEM0002-12 Section 9.23] |  |  |  |
| Inspect crankcase and front seal for leaks [JEM0002-12 Section 8.8] |  |  |  |
| Inspect distributor caps & rotors and adjust as needed) [JEM0002-12 Section 9.9] |  |  |  |
| Inspect ignition coils, electrical connections, and coil gap [JEM0002-12 Section 3.4] |  |  |  |
| Inspect safety wires for condition and orientation [AC43.13 Section 7-124] |  |  |  |

Propeller

| Inspection Item | Fail | Pass | Comments |
| --- | --- | --- | --- |
| Inspect spinner for damage and security [JEM0002-12 Section 8.7] |  |  |  |
| Inspect propeller for damage, nicks, and corrosion [JEM0002-12 Section 8.7] |  |  |  |
| Check propeller bolts for proper torque [JEM0002-12 Section 8.7] |  |  |  |
| Perform propeller and spinner tracking procedures [JEM0002-12 Section 9.2] |  |  |  |

Airframe

| Inspection Item | Fail | Pass | Comments |
| --- | --- | --- | --- |
| Engine and Nacelle Group |  |  |  |
| Inspect cowling for damage, cracks, chaffing, heat damage, security, and missing hardware [N/A] |  |  |  |
| Inspect Battery for damage and security of cables and test fault monitoring system [EarthX Lithium Battery User’s Manual] |  |  |  |
| Inspect voltage regulator and capacitor for damage and connection security and test regulator reset function [B&C XB1021-TM] |  |  |  |
| Inspect firewall pass-through points for damage and signs of leaks [AC43.13 Section 11-231] |  |  |  |
| Inspect welded components for cracks [SNX-SB-002] |  |  |  |
| Cabin and Cockpit Group |  |  |  |
| Inspect wiring harnesses underneath panel for damage and proper mount points [AC43.13 Section 11-32] |  |  |  |
| Inspect ground block underneath panel and verify integrity of all connections [AC43.13 Section 11-37] |  |  |  |
| Inspect fresh air vents for damage, missing hardware, and proper operation [N/A] |  |  |  |
| Verify proper function of all switches and circuit breakers located in the flight deck [AC43.13 Section 11-32] |  |  |  |
| Inspect seats cushions and wiring and verify operation of the heating elements [AC43.13 Section 9-64] |  |  |  |
| Inspect fuel cap and gasket for damage and pliability [AC43.13 Section 8-33] |  |  |  |
| Inspect fuel tank vent hoses, connections, and clamps for security, damage, and chaffing [AC43.13 Section 8-33] |  |  |  |
| Inspect fuel system underneath fuel tank for security and evidence of leaks [AC43.13 Section 8-31] |  |  |  |
| Inspect fuel drain for security, function, and evidence of leaks [AC43.13 Section 8-36] |  |  |  |
| Inspect exterior skins for damage, wrinkles, cracks, and loose hardware [AC43.13 Section 5-18] |  |  |  |
| Inspect windshield and canopy for cracks, scratches, and missing hardware [AC43.13 Section 3-40] |  |  |  |
| Verify canopy hinge, retention cable, latching mechanism, and spring tension and inspect for damage, looseness, and missing hardware [N/A] |  |  |  |
| Inspect interior side panels for damage and security then remove from aircraft along with seat cushions and seat pans [AC43.13 Section 9-62] |  |  |  |
| Inspect forward and aft wing attach points, hardware, angles, and skin for damage, cracks, missing hardware, and security [AC43.13 Section 5-18] |  |  |  |
| Inspect seatbelt and shoulder harness attach points for damage, deformities, and missing hardware [AC43.13 Section 9-64] |  |  |  |
| Inspect trim system (including balance spring) for function, routing, cable condition, and spring damage. [AC43.13 Section 7-149] |  |  |  |
| Inspect control stick assembly for wear, freedom of movement, security, and missing hardware. Lubricate pivot points with LPS-1 [N/A] |  |  |  |
| Inspect autopilot servos, pushrods, and mounts for damage, security, missing hardware, and freedom of movement. Also inspect autopilot servo wiring for security, damage, and proper routing [SkyView System Installation Guide Section 10] |  |  |  |
| Inspect com, transponder, and ADS-B antennas for security, hardware tightness, and antenna connections [AC43.13 Section 11-30] |  |  |  |
| Inspect GPS antennas for evidence of leakage, damage, and security [AC43.13 Section 11-30] |  |  |  |
| Inspect pitot/AOA lines for damage, security, and blockages [AC43.13 Section 12-53] |  |  |  |
| Inspect rudder pedals, mount blocks, return springs, and tension bars for damage, security, and missing hardware. Lubricate mount blocks with lithium grease [N/A] |  |  |  |
| Inspect rudder cables for damage, freedom of movement, proper tension, and routing [AC43.13 Section 7-144] |  |  |  |
| Inspect flap motor and flap drive tube for security, damage, missing hardware, and wiring. Lubricate flap drive tube with lithium grease [N/A] |  |  |  |
| Inspect elevator idler assembly for security, freedom of movement, slop, damage, and missing hardware. Lubricate with LPS-1 [N/A] |  |  |  |
| Inspect rear spar carry through for damage, missing hardware, and cracks [N/A] |  |  |  |
| Inspect baggage compartment for condition and damage. [AC43.13 Section 9-62] |  |  |  |
| Inspect elevator pushrod supports and snap bushings for wear, security, and condition. Lubricate with lithium grease [N/A] |  |  |  |
| Inspect welded components for cracks [SNX-SB-002] |  |  |  |
| Landing Gear Group |  |  |  |
| Remove wheel pants and gear leg fairings [N/A] |  |  |  |
| Inspect wheel pants for cracks, damage, and missing hardware [AC43.13 Section 9-2] |  |  |  |
| Inspect wheel pant mounting plate for damage, cracks, missing hardware, and overall alignment [AC43.13 Section 3-1] |  |  |  |
| Visually inspect wheel alignment and confirm findings with tire wear pattern [AC43.13 Section 9-2] |  |  |  |
| Replace upper gear leg hardware and axle hardware. Inspect holes for damage and elongation [SNX-SB-006] |  |  |  |
| Inspect brake lines and fittings for chafing, leaks, damage, and proper mounting hardware [AC43.13 Section 9-25] |  |  |  |
| Inspect brake pucks, caliper bores, and brake discs for abnormal wear, damage, cracks, and signs of leakage [AC43.13 Section 9-25] |  |  |  |
| Inspect Axle weldments and mounting tabs for damage, cracks, missing hardware, and elongation [AC43.13 Section 9-2] |  |  |  |
| Inspect tires for proper pressure, tread depth, and serviceability [AC43.13 Section 9-14] |  |  |  |
| Inspect wheels for cracks, damage, and missing hardware [AC43.13 Section 9-20] |  |  |  |
| Remove wheel bearings and old grease and repack with wheel bearing grease [AC43.13 Section 9-18] |  |  |  |
| Inspect interior brake lines, fuselage pass throughs, master cylinder, and brake handle for proper routing, signs of leaks, missing hardware, and chaffing. Verify parking brake valve operation and cable routing [AC43.13 Section 9-25] |  |  |  |
| Wing Group |  |  |  |
| Remove wing inspection plates [N/A] |  |  |  |
| Inspect wing skins for overall condition, damage, cracks, deformation, and wrinkling [N/A] |  |  |  |
| Inspect wing root weatherstripping for damage, cracks, security, and fitment [N/A] |  |  |  |
| Inspect landing lights for damage, missing hardware, lens condition, and aim [AC43.13 Section 11-1] |  |  |  |
| Inspect wing tips, lights, and fairings for damage, cracks, and missing hardware [AC43.13 Section 3-1] |  |  |  |
| Inspect wing wiring for damage, routing, chaffing, and security [AC43.13 Section 11-1] |  |  |  |
| Inspect aileron for freedom of movement, security, hinge pin hardware, damage, and counterweight hardware [N/A] |  |  |  |
| Inspect aileron pushrods and bell cranks. Lubricate bushings with LPS-1 [N/A] |  |  |  |
| Inspect flap for security, damage, hinge pin hardware, and pushrod hardware. Lubricate bushings with LPS-1 [N/A] |  |  |  |
| Inspect pitot/AOA tube for damage, security, and missing hardware [AC43.13 Section 12-51] |  |  |  |
| Inspect pitot heat controller for security, missing hardware, wire security, and damage [AC43.13 Section 12-55] |  |  |  |
| Inspect pitot/AOA lines for damage, security, routing, chaffing, and debris [AC43.13 Section 12-53] |  |  |  |
| Empennage Group |  |  |  |
| Remove empennage inspection covers, tail tips, and tail fairing panels [N/A] |  |  |  |
| Inspect stabilizer weatherstripping for damage, cracks, security, and fitment [N/A] |  |  |  |
| Inspect tail tips for damage, cracks, and security [AC43.13 Section 3-1] |  |  |  |
| Inspect vertical and horizontal stabilizer attachment points for security, missing hardware, and proper torque [N/A] |  |  |  |
| Inspect rudder cables, cable fairings, and attach points for damage, fraying, missing hardware, and security [AC43.13 Section 7-144] |  |  |  |
| Inspect elevator pushrod for security, damage, missing hardware, and clearance. Lubricate bushings with LPS-1 [N/A] |  |  |  |
| Inspect rudder for security, damage, hinge pin hardware, and freedom of movement [N/A] |  |  |  |
| Inspect elevator for security, damage, hinge pin hardware, and freedom of movement [N/A] |  |  |  |
| Inspect tailwheel spring and mount for damage, cracks, missing hardware, deformity, and security [AC43.13 Section 9-2] |  |  |  |
| Inspect tailwheel push rod for damage, missing hardware, and deformity [AC43.13 Section 9-2] |  |  |  |
| Inspect tailwheel and tailwheel assembly for damage, cracks, missing hardware, freedom of movement, and tire wear. Lubricate pivot and tailwheel bearing with LPS-1 [AC43.13 Section 9-14] |  |  |  |
| Inspect welded components for cracks [SNX-SB-002] |  |  |  |

Avionics

| Inspection Item | Fail | Pass | Comments |
| --- | --- | --- | --- |
| Perform annual ELT inspection [ACK E-04 ELT Installation Manual Section 10] |  |  | Controller battery expiration: Audio alert battery expiration: ELT battery expiration: |
| Perform Annual SV-BAT-320 Test [SkyView Installation Guide - Appendix A] |  |  |  |
| Perform AP servo test procedure [SkyView Installation Guide – Section 10] |  |  |  |
| Verify Dynon SkyView parameters are correct [AOH Section 2] |  |  |  |
| Complete ACS Service Bulletin SB24-01 [ACS Service Bulletin SB24-01] |  |  |  |

Wrap Up

| Inspection Item | Fail | Pass | Comments |
| --- | --- | --- | --- |
| Reinstall all inspection panels, fairings, wheel pants, cowling, and interior components [N/A] |  |  |  |
| Conduct engine post inspection test run [JEM0002-12 Section 9.1] |  |  |  |
| Perform AP servo test procedure [SkyView Installation Guide – Section 10] |  |  |  |
| Complete all required logbook entries and update maintenance tracer [N/A] |  |  |  |

| Signature and Certificate # |  | Date |
| --- | --- | --- |

General Notes: